# Boundary-Constrained Rare-Fault Triggering for Short-Horizon Hoister Fault-State Prediction

Abstract— Industrial hoisting systems need short-horizon state prediction before rare degradation becomes critical. In a private hoister overspeed dataset, standard time-series classifiers achieve high accuracy while missing the rare second-level degradation state. We formulate one-step future-state classification and propose SGTONetV6, which decouples conservative multiclass prediction from a calibrated rare-fault trigger constrained by transition boundaries and precursor states. On 27 files with 37,417 timestamps, SGTONetV6 achieves the best macro-F1 among tested models and recovers class 9 with F1 0.5556, while DLinear, TimesNet, iTransformer, PatchTST, and a conservative SGTO baseline all obtain class-9 F1 of 0.0000. Ablations show that rare override, boundary constraint, fallback thresholding, and patch-attentive rare context are necessary. The evidence supports short-horizon rare-fault recovery rather than broad multi-horizon superiority.

Keywords— hoisting system, overspeed fault, future-state classification, rare fault prediction, industrial time series.

# Introduction

Industrial hoisting systems are safety-critical assets, and overspeed degradation should be recognized before the fault state becomes unrecoverable. This digest studies a fixed-horizon future-state classification setting: given a current multivariate sensor window, predict the state label one step ahead. The task is difficult because the critical second-level degradation state is extremely rare. In the studied dataset, class 9 has only 185 timestamps out of 37,417, so high overall accuracy can coexist with complete rare-state collapse.

This failure mode appears in strong time-series classifiers. As shown later, DLinear, TimesNet, iTransformer, PatchTST, and a conservative SGTO baseline all obtain zero class-9 F1 under the same file-level split protocol. The main objective is therefore not simply to maximize accuracy. It is to recover the rare degradation state while keeping common-state predictions controlled.

The contribution is SGTONetV6, a shift-aware graph and trigger oriented network for short-horizon hoister state prediction. SGTONetV6 separates conservative multiclass classification from a rare-fault trigger. The trigger can override the base classifier only when its rare score exceeds a calibrated threshold and the sample satisfies transition-boundary and precursor-state constraints. This design targets the boundary region where the future label changes before the full sensor window looks like the future state.

# Task and Method

Let xt denote the multivariate sensor vector at time t. For a window length L, the input is Xt={xt−L+1,…,xt}, and the target is the future label yt+Δ. The digest focuses on Δ=1, with L=96 and a window step of 8. Labels are from the set {1,5,7,9,3}, representing stop, normal operation, first-level degradation, second-level degradation, and fault occurrence. The binary direct-fault indicator is dropped from the input to avoid leakage.

Fig. 1 summarizes SGTONetV6. A patch temporal encoder feeds a conservative future-state classifier and a rare-trigger branch. The rare branch computes patch-attentive context because rare degradation evidence can appear in only a short part of the input. At inference, the final decision follows

y=9,s9≥τ, b=1, yt∈{5,7},ybase,otherwise,

where s9 is the rare score, τ is the calibrated threshold, b is the boundary flag, and yt is the current state. This rule prevents the rare trigger from firing globally and makes the method different from using only a class-weighted loss or a larger backbone.

SGTONetV6 decouples conservative future-state classification from a calibrated rare-fault trigger constrained by boundary and precursor semantics.

# Experiments

The dataset contains 27 files, 37,417 timestamps sampled every 4 seconds, and five state labels. The class counts are 10,959 for stop, 15,695 for normal operation, 5,364 for first-level degradation, 185 for second-level degradation, and 5,214 for fault occurrence. Evaluation uses file-level train/validation/test splits with split seeds 14, 22, and 30. The primary metrics are macro-F1, balanced accuracy, fault macro-F1, and class-9 precision/recall/F1.

Main comparison on the Δ=1 future-state classification task. Values are three-split means.

| Method | Acc. | Macro-F1 | Fault F1 | C9 F1 |
| --- | --- | --- | --- | --- |
| SGTONetV6 | 0.7102 | 0.6233 | 0.5411 | 0.5556 |
| iTransformer | 0.8175 | 0.6185 | 0.4615 | 0.0000 |
| DLinear | 0.7895 | 0.5961 | 0.4426 | 0.0000 |
| TimesNet | 0.7958 | 0.5893 | 0.4275 | 0.0000 |
| PatchTST | 0.7559 | 0.5687 | 0.4194 | 0.0000 |
| SGTONetV4 | 0.7630 | 0.5605 | 0.3999 | 0.0000 |

Table 1 shows that SGTONetV6 has the best macro-F1 and fault macro-F1 among the tested models, but not the best accuracy. This is expected: the method is optimized for rare-fault recovery rather than top-line accuracy. iTransformer has the highest accuracy, yet its class-9 F1 is zero. Fig. 2 isolates this rare-class behavior.

Class-9 precision, recall, and F1. Baselines collapse to zero class-9 F1, while SGTONetV6 recovers the rare degradation state.

# Ablation and Scope

SGTONetV6 ablation results on Δ=1.

| Variant | Macro-F1 | Bal. Acc. | C9 F1 |
| --- | --- | --- | --- |
| Full SGTONetV6 | 0.6233 | 0.6731 | 0.5556 |
| No precursor constraint | 0.6139 | 0.6725 | 0.5101 |
| Mean rare context | 0.5848 | 0.6654 | 0.2317 |
| No fallback prior | 0.5830 | 0.6397 | 0.3556 |
| No rare override | 0.5113 | 0.5568 | 0.0000 |
| No boundary constraint | 0.4550 | 0.5469 | 0.0158 |

Table 2 supports the mechanism. Removing rare override collapses class-9 F1 to zero, which means the conservative base classifier alone does not solve rare-state recovery. Removing the boundary constraint causes uncontrolled triggering and reduces class-9 F1 to 0.0158. Replacing patch-attentive rare context with mean context also weakens rare recovery, indicating that local evidence inside the window matters.

Ablation study. Rare override, boundary constraint, fallback thresholding, and patch-attentive context all contribute to rare-fault recovery.

The current result should be scoped carefully. At Δ=3, PatchTST reaches macro-F1 0.5877 and class-9 F1 0.1919, while SGTONetV6 reaches macro-F1 0.5006 and class-9 F1 0.1070. Thus, the current configuration supports short-horizon rare-fault recovery, not general multi-horizon superiority.

# Conclusions and Future Works

This digest identifies rare-boundary collapse as a practical failure mode in short-horizon hoister fault-state prediction. SGTONetV6 addresses it by combining conservative multiclass prediction with a boundary-constrained rare-fault trigger. On the studied industrial dataset, the method recovers class 9 with F1 0.5556 while tested baselines obtain zero class-9 F1. Future work will validate the method on additional industrial assets, improve horizon-specific trigger calibration, and evaluate deployment-level false-alarm costs.

9 H. Wu, T. Hu, Y. Liu, H. Zhou, J. Wang, and M. Long, “TimesNet: Temporal 2D-variation modeling for general time series analysis,” in Proc. ICLR, 2023.

Y. Liu, T. Hu, H. Zhang, H. Wu, S. Wang, L. Ma, and M. Long, “iTransformer: Inverted transformers are effective for time series forecasting,” in Proc. ICLR, 2024.

Y. Nie, N. H. Nguyen, P. Sinthong, and J. Kalagnanam, “A time series is worth 64 words: Long-term forecasting with transformers,” in Proc. ICLR, 2023.

A. Zeng, M. Chen, L. Zhang, and Q. Xu, “Are transformers effective for time series forecasting?” in Proc. AAAI, 2023.